Code for 1-ReLU Bandit interpretation and Microkinetic analysis

1.Aim:

The purpose of the script above is to extract the weights and biases of the 1-ReLU Bandit model and use them for subsequent microkinetic modeling analysis. The usage of the script can be summarized in five steps; below we provide a functional description of what the script does in each step.

2.Step 1-get the Weight and Bias from the 1-ReLU Bandit model

Script “model_export.py” covers (i) how the 1-ReLU Bandit model is trained on the input data and used to predict under new test conditions, and (ii) how the trained model’s weights and biases are extracted after model construction. Our 1-ReLU Bandit model incorporates 1- to 4-order terms of x* and contains 2048 neurons in the hidden layer; therefore, the output y= W1×ReLU W0× x*+ b0+b1 can be expressed by the above equation. Here, b0 is a 494 × 1 vector, W0 is a 494 × 2048 matrix, b1 is a 1 × 1 scalar, and W1 is a 2048 × 1 matrix, all of which are exported via the corresponding code modules.

3.Step 2-calculate the coefficient Ci and Ai plot for each input x0

After the model is constructed, we can characterize a potential-energy-surface-like relationship between the output y kobs and the input x* (with the 1- to 4-order terms, total 494 features, To avoid ambiguity, following xi† here denotes the i-th number feature among the 494 input features; in the derivations of this section, xi† does not only refer exclusively to a first-order variable). With a linearization approximation, it can be assumed that:

y ≈ i=1494xi†×Ci+b

via a Jacobian approximation, it can be written as:

∂y∂xi†= j=12048wij(0)× 1zj>0× wj(1)

j here denotes the j-th number of the neurons (with total 2048) and this approach enables evaluation of partial derivatives at arbitrary input conditions x0, thereby facilitating direct linkage to the subsequent microkinetic modeling.

To implement the above functionality and enable visualization, this step consists of 3 scripts. The script “Calculate Ci.py” comprises four sub-steps: (1) loading the weights and biases form the trained 1-ReLU Bandit model, (2) generating polynomial features (1- to 4- order), (3) generating feature names for each polynomial term, and (4) computing Ci. Meanwhile, the initial condition x0 can be specified by the user as an 8-dimensional vector, corresponding to the (0 to 1 normalized) concentrations of [Phenol, TFE, 1,1,1,3,3,3-Hexafluoro-2-methyl-2-propanol, TBAPF6, CsPF6, TMAPF6, Perylene]. To probe the influence of CO2, we sweep the CO2 concentration from 0 to 1 with a step partial pressure of 0.01. This enables extraction of the Ci partial-derivative coefficients for all 494 1-order to 4-order features at arbitrary CO2 concentration, which are then used for downstream analysis.

We are first and foremost interested in the Axi terms as following

Axi=∂2kobs*∂xi∂[CO2]CO2+∂kobs*∂xi

Axi can be interpreted as the extent of changes in kobs* when xi changes under a certain fixed CO2 concentration, the script “Get first order and second order Ci.py” enables precise extraction of the partial-derivative information associated with the first- (xi) and CO2-dependent second-order terms (xixCO2), while “Plot Axi equation.py” visualizes these results.

4. Step 3-statistically analysis the partial-derivative coefficients at 5% and 100% CO2

During Step 2, we can obtain partial-derivative information extracted at arbitrary x0 conditions across different CO2 concentrations. To render the results statistically meaningful, we systematically examined the Axi variations derived from the top 20 high CO2RR activity from experimentally measured x0 conditions (include 10 conditions from 5% CO2 and 10 from 100% CO2), and calculate the corresponding average values (Mean) and standard deviations (SD). The script “Calculate Mean and SD.py” performs these calculations, while “Plot-Axi equation-Mean and SD.py” visualizes the results.

The above framework further enables quantification of the synergistic contributions arising from any pair of variables xiand xj, where xiand xj correspond to proton donors, cations, and redox mediators. That is the term for Bxixj shown as follow:

Bxixj=∂2kobs*∂xi∂xj

We extracted the pairwise partial-derivative coefficients Bxixjunder CO2 = 5% and 100%, Likewise, we performed statistics over the same top 20 conditions and computed the corresponding average values (Mean) and standard deviations (SD). For the 5% CO2 case, “Calculate Mean and SD-for 0.05Ci.py” and “Contribution-heat map-0.05Ci.py” perform the extraction and visualization of Bxixj, respectively. For the 100% CO2 case, “Calculate Mean and SD-for 1.00Ci.py” and “Contribution-heat map-1.00Ci.py” carry out the extraction and visualization of Bxixj, respectively.

5. Step 4-Normalized unit

To account for the order-of-magnitude differences in concentrations under practical measurement conditions: proton donors (0 to 1 M), cation electrolytes (0 to 0.1 M), and redox mediators (0 to 1.25 mM), we calculated Axi and Bxixj terms by concentration to consistent Molar unit. Under this normalization, Axi has units of M-1•s-1, and Bxixj has units of M-2• s-1. The scripts “Normalized unit-Plot Axi equation-Mean and SD.py”, “Normalized unit-Bxixj-heat map-0.05Ci.py”, and “Normalized unit-Bxixj-heat map-1.00Ci.py” implement this normalization and visualize the resulting data.

6. Step 5-mathematical derivation and relate Axiand Bxixjto Microkinetic model

After establishing a microkinetic model for the molecular electrocatalysis of the precatalyst  [FeIIITPP]Cl, we further link the Axi and Bxixj terms obtained from the 1-ReLU Bandit model to the microkinetic framework.

-RT2Axikobs*=-RT∂ln1+δγ∂xi,med+∂Ea,TLS∂xi,med+∂∆Gpre∂xi,med

-RTBxixjkobs*-Axi2kobs*Axj2kobs*=∂2Ea,TLS∂xi∂xj+∂2∆Gpre∂xi∂xj

We computed and analyzed the left-hand-side terms of the two equations above, given that kobs*varies with CO2 concentration, we fitted the experimental kobs* - CO2 dependence using the Langmuir-type model, which is broadly consistent with the proposed following equation:

kobs* = 5.24×104× 9.8 × P(CO2)1+ 9.8 × P(CO2)

The resulting concentration-dependent kobs *values were then assigned to their corresponding CO2 concentrations. The script “Axi related items calculation.py” computes and visualizes the-RT2Axikobs* terms, and the script “Calculate Bxixj related items-5% CO2.py” and script “plot for Bxixj related items-5% CO2.py” computes and visualizes the -RTBxixjkobs*-Axi2kobs*Axj2kobs* terms under 5% CO2, while the script “Calculate Bxixj related items-100% CO2.py” and script “plot for Bxixj related items-100% CO2.py” computes and visualizes the -RTBxixjkobs*-Axi2kobs*Axj2kobs* terms under 100% CO2.

It should be noted that the ∂2Ea,TLS∂xi∂xj and ∂2∆Gpre∂xi∂xj term can be interpreted as the “cross-talk” term that represents how reaction condition xi and xj collaboratively affect the TLS’s activation barrier and the free energy change of the pre-equilibrium, respectively. Following the aforementioned argument that ∆Gpre is expected to be sensitive towards CO2 while Ea,TLS much less likely so, comparing the calculated values at different CO2 values will help to differentiate multi-component synergy in TLS from the one of pre-equilibrium. Thus we also analysis the difference between the -RTBxixjkobs*-Axi2kobs*Axj2kobs* term from 5% CO2 and 100% CO2, and the script “Bxixj related term-CO2 difference.py” computes and visualizes this.