Part I: Introduction

In this section, we will establish a microkinetic model of the molecular electrocatalysis of pre-catalyst [FeIIITPP]Cl, and further connect the terms Ai and Bij obtained from the 1-ReLU bandit to the microkinetic models. The overall aim to provide a physical “interpretation” of Ai and Bij in the context of molecular electrocatalysis. Moreover, as the microkinetic model discussed below is quite common and presents a general scheme in molecular catalysis, the developed methodology is proposed to serve as an example for future researchers to analyze information from interpretable models of reinforcement learning in self-driving laboratories.

Above shows the commonly reported catalytic cycle of FeTPP in electrocatalytic CO2 reduction1. The general assumption is that after outer-sphere charge transfer to generate the Fe0 dianion (Fe0), the steps of CO2 binding and the first proton transfer (PT) are both under pre-equilibrium, and the turnover limiting step (TLS) is the second PT that leads to a FeII carbonyl adduct. In the above diagram, we illustrated how cations, proton donors, and electron mediator may affect the catalytic cycle. Different cations will change the thermodynamic energetics of anionic intermediates. The addition of (mixed) proton donors will affect the thermodynamics of the first PT step and the kinetics in the TLS. Lastly, the redox mediator will affect the partition between the Fe0 and FeI anion radical (FeI), and more importantly will change the effective thickness of diffusion-reaction layer near the electrode as recently outlined.2

Part II: Mathematical equations of the microkinetic model

In the above catalytic cycle, the pre-equilibrium of outer-sphere charge transfer between FeI and Fe0 is governed by the Nernst equation,

E=Ecat-RTFlnFe0FeI                                                                                                                        (1)

Here Fe0 and FeI are the concentrations of Fe0 and FeI species in the solution in the unit of M, respectively; Ecat is the standard potential of FeI/0 redox couple; E is the solution’s electrochemical potential at a certain steady-state, which is equivalent to the solution’s Fermi level collectively determined by all of the redox couples in the solution.

The CO2-binding step is presented as a pre-equilibrium,

K1=FeCO2C0Fe0CO2                                                                                                                                         (2)

Here K1 is the dimensionless equilibrium constant; FeCO2 and CO2 are the concentrations of FeTPP-CO2 adduct and CO2, respectively; C0 = 1 M serves as a constant to render K1 dimensionless.

The first PT step is also presented as a pre-equilibrium,

K2=FeCOOHA-FeCO2HA                                                                                                                                  (3)

Here K2 is the dimensionless equilibrium constant; FeCOOH, A-, and HA are the concentrations of monoprotonated FeTPP-CO2 adduct, the conjugated base and acid of the proton donor, respectively.

The turnover-limiting second PT is presented by the following kinetic expression, assuming this TLS is an elemental step with well-defined reaction order.

r=kobsCcat=kTLSHAFeCOOH                                                                                                      (4)

kobs=kTLSHAFeCOOHCFe                                                                                                                    (5)

Here r is the reaction rate of the net CO2-reduction process in the unit of M−1•s−1; kobs is the observable pseudo-first-order rate constant in the unit of s−1; Ccat is the net concentration of FeTPP species in the solution; kTLS is the second-order rate constant of this specific TLS.

Combining equation (1) to (5), we have,

kobs=kTLSK1K2CO2C0FeICcatHA2A-exp-FRTE-Ecat                                                            (6)

In the presence of redox mediator, the effective observable pseudo-first-order rate constant kobs*, measured experimentally from the foot-of-wave analysis and in the unit of s−1, can be described below,2

kobs*=1+δγkobs=1+δγkTLSK1K2CO2C0FeICcatHA2A-exp-FRTE-Ecat            (7)

The above equation assumes that the standard potential of the redox mediator Emed≈Ecat, or effectively,2

Ke=expFRTEcat-Emed≈1                                                                                                          (8)

Here δ=DmedDcat is the ratio of diffusion coefficients for the mediator (Dmed) and Fe catalyst (Dcat); γ=CmedCcat is the ratio of net concentrations for the mediator (Cmed) and the Fe catalyst (Ccat), following the definitions in literature.2

In summary, equation (7) establishes the mathematical expression of the effective observable pseudo-first-order rate constant kobs* in the context of the microkinetic model.

Part III: Assumptions about how cations, proton donors, and mediator affects terms in the microkinetic model

Equation (7) suggests that there are three major components that affecting kobs*: (1) the mediator-induced increase of diffusion-reaction layer in term 1+δγ; (2) the kinetic factor in term kTLS; (3) the thermodynamic factor of the pre-equilibrium steps in the rest of equation. Correspondingly, we can simply the equation (7) as,

kobs*=1+δγkTLSKpre                                                                                                                           (9)

in which,

Kpre=K1K2CO2C0FeICcatHA2A-exp-FRTE-Ecat                                                                 (10)

We assume that the cations may affect both Kpre and kTLS, because the electrostatic interactions between the cation and negatively charged catalytic molecules will change both the thermodynamics in the pre-equilibrium steps and the kinetic barrier in the TLS. We assume that proton donors may also affect both Kpre and kTLS, because the proton donor’s acidity and the non-covalent intermolecular interactions between the catalyst and proton donor will also affect both the thermodynamics in pre-equilibrium and the kinetic barrier in the TLS.

Moreover, we assume that the redox mediator may affect Kpre and kTLS in addition to 1+δγ. The dependence on redox mediator for Kpre and kTLS can be argued in two separate reasons: (1) Recent reports suggest that the π-π interactions between the conjugated FeTPP and polyaromatics may induce conformational changes for individual catalyst molecule hence changes in electrocatalytic activities;3 (2) Since standard potential of redox mediator Emed can hardly be exactly the same as FeTPP’s Ecat, any slight difference between Emed and Ecat will vary the solution’s steady-state Fermi level E, particularly given that the concentration of redox mediator Cmed tends to be in excess in comparison to the catalyst’s concentration Ccat.

Part IV: Derivation of Ai in the context of microkinetic model

As discussed in the main text, Ai obtained from the 1-ReLU bandit model, in the unit of M−1•s−1, is defined as,

Ai=∂2kobs*∂xi∂[CO2]CO2+∂kobs*∂xi                                                                                                          (11)

Here xi is defined as the concentrations of cations, proton donors, and mediators.

Empirically, Ai can be interpreted as the extent of changes in kobs* when xi changes under a certain fixed CO2 concentration. Here we will analytical derive Ai based on the above discussions and attempt providing a more fundamental interpretation towards Ai.

The assumptions listed in the previous section state that redox mediator affect 1+δγ, Kpre and kTLS while proton donor and cations only affect  Kpre and kTLS. In order to separate different scenarios, in the rest of this discussion we define xi,med, equivalent to Cmed, as the concentration of redox mediator, while xi,+ as the concentration of a proton donor or cation.

First, we consider the case of a redox mediator xi,med,

∂kobs*∂xi,med=kobs*∂ln1+δγ∂xi,med+∂lnkTLS∂xi,med+∂lnKpre∂xi,med                                                               (12)

∂kobs*∂[CO2]=kobs*[CO2]                                                                                                                                         (13)

∂2kobs*∂xi,med∂[CO2]=kobs*[CO2]∂ln1+δγ∂xi,med+∂lnkTLS∂xi,med+∂lnKpre∂xi,med                                              (14)

Therefore,

Ai=2kobs*∂ln1+δγ∂xi,med+∂lnkTLS∂xi,med+∂lnKpre∂xi,med                                                                   15a

Or equivalently,

Ai2kobs*=∂ln1+δγ∂xi,med+∂lnkTLS∂xi,med+∂lnKpre∂xi,med                                                                            15b

Second, in the case of proton donor or cations xi,+, we similarly have,

Ai=2kobs*∂lnkTLS∂xi,++∂lnKpre∂xi,+                                                                                                16a

Ai2kobs*=∂lnkTLS∂xi,++∂lnKpre∂xi,+                                                                                                        16b

Part IV: Discussion and interpretation of Ai in molecular electrocatalysis

Equations (16a/b) deploys a term Ai2kobs*, which can be interpreted as ½ of the differential relative improvement in kobs* upon the change of concentration xi,med or xi,+. Our derivation indicates that Ai2kobs* could be linearized into three terms that concerning the expansion of diffusion-reaction layer (∂ln1+δγ∂xi,med), the kinetics of TLS (∂lnkTLS∂xi,med or ∂lnkTLS∂xi,+), and the thermodynamics of the pre-equilibrium (∂lnKpre∂xi,med or ∂lnKpre∂xi,+). Below we individually discuss these three terms and provide our interpretation of their physical meanings.

Term I: ∂ln1+δγ∂xi,med

Based on equation (7) and the prior discussion of mediated electrocatalysis2, there first term in equation (15a/b) can be derived as,

∂ln1+δγ∂xi,med=1DcatDmedCcat+xi,med                                                                                                       (17)

This suggests that near local optima of kobs* there is diminishing returns in relative kobs* improvement, when constantly increasing the concentration of redox mediator xi,med.

Term II: ∂lnkTLS∂xi,med or ∂lnkTLS∂xi,+

Under the transition-state theory, the TLS can be expressed as,

kTLS=ATLSexp-Ea,TLSRT                                                                                                                    (18)

Here ATLS and Ea,TLS are the pre-factor and activation energy of the TLS’s transition state.

Because Ai is defined as the differential increase of kobs* upon increase of xi,med or xi,+, in a first-order approximation we are eligible to assume that the ATLS is mostly unaltered and it is the Ea,TLS that exerts the biggest impacts on kobs*. Upon such an assumption,

∂lnkTLS∂xi,med=-1RT∂Ea,TLS∂xi,med                                                                                                                   (19a)

∂lnkTLS∂xi,+=-1RT∂Ea,TLS∂xi,+                                                                                                                   (19b)

Term III: ∂lnKpre∂xi,med or ∂lnKpre∂xi,+

Based on the definition in equation (10), Kpre represents the thermodynamic equilibrium constant collectively for all the pre-equilibrium steps. If we define ∆Gpre=-RTlnKpre as the Gibbs free energy change from FeI to FeCOOH during the pre-equilibrium, we have,

∂lnKpre∂xi,med=-1RT∂∆Gpre∂xi,med                                                                                                                   (20a)

∂lnKpre∂xi,+=-1RT∂∆Gpre∂xi,+                                                                                                                   (20b)

A summary of the discussion presented above leads to a revised relationship for Ai obtained from the 1-ReLU bandit model.

For redox mediator, the relationship is,

-RT2Aikobs*=-RT∂ln1+δγ∂xi,med+∂Ea,TLS∂xi,med+∂∆Gpre∂xi,med                                                                    (21a)

For cations and proton donors, the relationship is,

-RT2Aikobs*=∂Ea,TLS∂xi,++∂∆Gpre∂xi,+                                                                                                           (21b)

From the fundamental catalytic perspective, our analysis that -RT2Aikobs*, whose unit is kJ•M−2 (or kcal•M−2), presents the differential sensitivity of activation energy during the TLS (∂Ea,TLS∂xi,med and ∂Ea,TLS∂xi,+) and the free energy change of pre-equilibrium (∂∆Gpre∂xi,med and ∂∆Gpre∂xi,+) upon changes of xi,med and xi,+.

For redox mediator, the contribution of diffusion-reaction layer in -RT2Aikobs* can be calculated from equation (17). Therefore, it is feasible to separate the contribution from diffusion-reaction layer and account analyze the sensitivity of Ea,TLS and ∆Gpre towards proton donors, cations, and redox mediators. Moreover, as the event of CO2 binding takes place during the pre-equilibrium stage, this suggests that ∆Gpre should have a strong dependence on CO2 concentration CO2, while the Ea,TLS should be mostly insensitive towards CO2. Correspondingly, the changes in -RT2Aikobs* when increasing CO2 partial pressure to 0 to 100% (0.23 M) approximately present the differential sensitivity in pre-equilibrium’s free energy change (∂∆Gpre∂xi,med and ∂∆Gpre∂xi,+), while the remaining component represents the differential sensitivity in TLS’s activation energy (∂Ea,TLS∂xi,med and ∂Ea,TLS∂xi,+) after subtracting the value of -RT∂ln1+δγ∂xi,med in the case of redox mediators.

Part V: Derivation of Bij in the context of microkinetic model

As discussed in the main text, Bij obtained from the 1-ReLU bandit model, in the unit of M−2•s−1, is defined below when xi and xj are proton donors, cations, and redox mediator,

Bij=∂2kobs*∂xi∂xj                                                                                                                                              22

Because redox mediator has different mathematical expressions in comparison with proton donors, in general Bij has three variations in different scenarios: (1) both xi and xj are redox mediator (xi=xj=xi,med); (2) one of xi and xj is redox mediator and the other is not; (3) neither xi and xj is redox mediator. Below we will derived the expression of Bij under each scenario.

When both xi=xj=xi,med,

Bij=kobs*∂ln1+δγ∂xi,med+∂lnkTLS∂xi,med+∂lnKpre∂xi,med2+∂2ln1+δγ∂xi,med2+∂2lnkTLS∂xi,med2+∂2lnKpre∂xi,med2                                                                                                            (23a)

Or equivalently,

Bijkobs*=Ai2kobs*2+∂∂xi,medAi2kobs*                                                                                                  (23b)

When one of xi and xj is redox mediator and the other is not (xi=xi,med and xj=xj,+),

Bij=kobs*∂ln1+δγ∂xi,med+∂lnkTLS∂xi,med+∂lnKpre∂xi,med∂lnkTLS∂xj,++∂lnKpre∂xj,++∂2lnkTLS∂xi,med∂xj,++∂2lnKpre∂xi,med∂xj,+                                                                                                           (24a)

Or equivalently,

Bijkobs*=Ai2kobs*Aj2kobs*+∂2lnkTLS∂xi,med∂xj,++∂2lnKpre∂xi,med∂xj,+                                                                       (24b)

When neither xi and xj is redox mediator (xi=xi,+ and xj=xj,+), we have,

Bij=kobs*∂lnkTLS∂xi,++∂lnKpre∂xi,+∂lnkTLS∂xj,++∂lnKpre∂xj,++∂2lnkTLS∂xi,+∂xj,++∂2lnKpre∂xi,+∂xj,+                                                                                                             25a

Or equivalently,

Bijkobs*=Ai2kobs*Aj2kobs*+∂2lnkTLS∂xi,+∂xj,++∂2lnKpre∂xi,+∂xj,+                                                                           (25b)

Evaluating equations (23a/b), (24a/b), and (25a/b) leads to the conclusion that equation (25b) represents a general expression for all the 3 scenarios concerned in this discussion.

Part VI: Discussion and interpretation of Bij in molecular electrocatalysis

Incorporating equations (19a/b) and (20a/b) into equation (25b) and leads to an interesting expression whose unit is kJ•M−3 (or kcal•M−3),

-RTBijkobs*-Ai2kobs*Aj2kobs*=∂2Ea,TLS∂xi∂xj+∂2∆Gpre∂xi∂xj                                                                           (26)

The above equation indicates that it is feasible to extract the synergistic information of reaction mechanism. ∂2Ea,TLS∂xi∂xj and ∂2∆Gpre∂xi∂xj can be interpreted as the “cross-talk” term that represents how reaction condition xi and xj collaboratively affect the TLS’s activation barrier and the free energy change of the pre-equilibrium, respectively. Following the aforementioned argument that ∆Gpre is expected to be sensitive towards CO2 while Ea,TLS much less likely so, comparing the calculated values of equation (26) at different CO2 values will help to differentiate multi-component synergy in TLS from the one of pre-equilibrium.

References:

(1) Costentin, C.; Savéant, J.-M. Towards an intelligent design of molecular electrocatalysts. Nature Reviews Chemistry 2017, 1, 0087, Perspective. DOI: 10.1038/s41570-017-0087.

(2) Harvey, C. M.; Costentin, C. Redox Mediated Molecular Chemical Catalysis of Electrochemical Reactions: A Co-Catalytic Boosting Effect in CO2 Electroreduction Catalyzed by an Iron Porphyrin. ACS Catalysis 2025, 15 (17), 14927-14937. DOI: 10.1021/acscatal.5c03381.

(3) Branch, K. L.; Johnson, E. R.; Nichols, E. M. Porphyrin Aggregation under Homogeneous Conditions Inhibits Electrocatalysis: A Case Study on CO2 Reduction. ACS Central Science 2024. DOI: 10.1021/acscentsci.4c00121.